IC Control 500

Flexible RFID System Mainboard

| iDTRONIC GmbH Donnersbergweg 1 67059 Ludwigshafen Germany/Deutschland Phone: +49 621 6690094-0 Fax: +49 621 6690094-9 E-Mail: info@idtronic.de Web: idtronic.de | Issue 1.6 – 05. October 2020 – Subject to alteration without prior notice. © Copyright iDTRONIC GmbH 2020 Printed in Germany |
| --- | --- |

Contents

1	Function Description	4

1.1	Overview	4

1.2	Mainboard	4

1.3	Power Supply Circuitry on Main Board	4

1.4	SD/TF Card Slot	4

1.5	Interfaces to RFID electronics	4

1.6	Other Interfaces and Hardware	4

1.7	Other Interfaces and Hardware	5

1.7.1	RTC with DS3231	5

1.7.2	Header for external display SBC-LCD20x4	5

1.8	Protected Outputs	5

1.9	Protected Inputs	5

1.10	Miscellaneous	6

1.11	Illustration of Mainboard with 4 different Versions of RFID	6

2	Housing	8

3	Connections on PCB	9

3.1	Odd numbered PINs on DDR Connector of Raspberry Compute Module	9

3.2	Even numbered PINs on DDR Connector of Raspberry Compute Module	11

4	Ressources	14

5	Revision History	15

6	Technical Data ·  Preliminary	16

# Function Description

## Overview

This is intended to be a new mainboard for up to 2 RFID modules or external optical code readers.

The main circuitry is the Raspberry Compute Module 3+.

The main interfacing is the Ethernet + USB-Hub IC LAN9514i.

The Realtek 8188-EU WiFi module provides WiFi.

For other hardware 3 USB ports are available.

## Mainboard

Raspberry Pi Compute Module 3 B+

2 × OEM-DES-M900-TTL or

2 × Bluebox M900-TTL or

1 × Bluebox M950-TTL

14 × Digital Outputs protected with NPN transistors (transistor array)

4 × Digital Inputs protected with opto-coupler

## Power Supply Circuitry on Main Board

Power supply connector socket for 5.5/2.5 mm round plugs.

The Main Board shall provide power supply for the Raspberry Compute 3+

+3.3 Vdc:		VBAT, 3V3, GPIO0-27_VREF, GPIO28-45V_REF + HF-RFID	4 A

+1.8 Vdc:		1V8, SDX_VDD							200 mA

+5 Vdc (external):	Only for UHF RFID, 							2 A

Important Note

Please pay attention to the information in chapter “7.1 Supply Sequencing” in the data sheet “Raspberry Pi Compute Modules 3 (2019).pdf”.

## SD/TF Card Slot

SD/TF card slot to GPIO8, GPIO9, GPIO10, GPIO11 (Pins 27, 29, 33, 35)

## Interfaces to RFID electronics

2 × TTL:

GPIO14 (configuration: ALT0_TXD0) = RFID1_TXD

GPIO15 (configuration: ALT0_RXD0) = RFID1_RXD

GPIO32 (configuration: ALT5_TXD1) = RFID2_TXD

GPIO33 (configuration: ALT5_RXD1) = RFID2_RXD

## Other Interfaces and Hardware

WiFi and Ethernet + USB hub with LAN9514i

The LAN9514i provides 5 USB ports.

Its port USB0 is connected directly to the Raspberry Compute Modules USB port.

Its port USB4 is connected directly to an Realtek 8188-EU WiFi module onboard.

The other 3 USB ports are used freely. This is the job of our configuration and our application software.

## Other Interfaces and Hardware

Display-Interface on I²C bus together with RTC. Both devices can be selected by the bus address.

### RTC with DS3231

I2C_SDA1 (Pin 9)

I2C_SCL1 (Pin 11)

### Header for external display SBC-LCD20x4

## Protected Outputs

6 × for LEDs

4 × for Turnstile Control

Hardware suggestion: 	2 Transistor Arrays of 7 ULN-2003A = 14 Outputs, 4 remain unused

## Protected Inputs

4 × for PS2501L-4

## Miscellaneous

## Illustration of Mainboard with 4 different Versions of RFID

Mainboard PCB with 2 × OEM-DES-M900-TTL

Mainboard with 2 × UHF M800-TTL

Mainboard with 2 × UHF M900-TTL

Mainboard with 1 × UHF M950-TTL

# Housing

It is intended that this mainboard will be built into a longer version of this housing:

# Connections on PCB

SD0 reserved for internal flash memory

## Odd numbered PINs on DDR Connector of Raspberry Compute Module

| Pin # | Raspberry Designation | Configuration | Function Designation = Net Name | Peripheral Device, Remarks |
| --- | --- | --- | --- | --- |
| 1 | GND |  | GND |  |
| 3 | GPIO0 | — | — | DO NOT CONNECT! |
| 5 | GPIO1 | — | — | DO NOT CONNECT! |
| 7 | GND |  | GND |  |
| 9 | GPIO2 | ALT0_SDA1 | I2C_SDA1 | RTC + Display |
| 11 | GPIO3 | ALT0_SCL1 | I2C_SCL1 | RTC + Display |
| 13 | GND |  | GND |  |
| 15 | GPIO4 | — | — | — |
| 17 | GPIO5 | — | — | — |
| 19 | GND |  | GND |  |
| 21 | GPIO6 | — | — | — |
| 23 | GPIO7 | OUTPUT | — | — |
| 25 | GND |  | GND |  |
| 27 | GPIO8 | OUTPUT | SPI0_CE0_N | SD-Card Enable |
| 29 | GPIO9 | SPI0_MISO | SPI0_MISO | SD-Card MISO |
| 31 | GND |  | GND |  |
| 33 | GPIO10 | SPI0_MOSI | SPI0_MOSI | SD-Card MOSI |
| 35 | GPIO11 | SPI0_SCLK | SPI0_SCLK | SD-Card CLK |
| 37 | GND |  | GND |  |
| 39 | GPIO0-27_VDD | — | 3V3 | +3.3 Vdc |
| KEY |  |  |  |  |
| 41 | GPIO28-45_VDD | — | 3V3 | +3.3 Vdc |
| 43 | GND |  | GND |  |
| 45 | GPIO12 | — | — | — |
| 47 | GPIO13 | — | — | — |
| 49 | GND |  | GND |  |
| 51 | GPIO14 | ALT0_TXD0 | RFID1_TXD | UART0, RFID1 |
| 53 | GPIO15 | ALT0_RXD0 | RFID2_RXD | UART0, RFID1 |
| 55 | GND |  | GND |  |
| 57 | GPIO16 | — | — | SD0 reserved! |
| 59 | GPIO17 | — | — | — |
| 61 | GND |  | GND |  |
| 63 | GPIO18 | — | — | — |
| 65 | GPIO19 | — | — | — |
| 67 | GND |  | GND |  |
| 69 | GPIO20 | — | — | — |
| 71 | GPIO21 | — | — | — |
| 73 | GND |  | GND |  |
| 75 | GPIO22 | — | — | — |
| 77 | GPIO23 | — | — | — |
| 79 | GND |  | GND |  |
| 81 | GPIO24 | — | — | — |
| 83 | GPIO25 | — | — | — |
| 85 | GND |  | GND |  |
| 87 | GPIO26 | — | — | — |
| 89 | GPIO27 | — | — | — |
| 91 | GND |  | GND |  |
| 93 | DSI0_DN1 | — | — | — |
| 95 | DSI0_DP1 | — | — | — |
| 97 | GND |  | GND |  |
| 99 | DSI0_DN0 | — | — | — |
| 101 | DSI0_DP0 | — | — | — |
| 103 | GND |  | GND |  |
| 105 | DSI0_CN | — | — | — |
| 107 | DSI0_CP | — | — | — |
| 109 | GND |  | GND |  |
| 111 | HDMI_CLK_N | — | — | — |
| 113 | HDMI_CLK_P | — | — | — |
| 115 | GND |  | GND |  |
| 117 | HDMI_D0_N | — | — | — |
| 119 | HDMI_D0_P | — | — | — |
| 121 | GND |  | GND |  |
| 123 | HDMI_D1_N | — | — | — |
| 125 | HDMI_D1_P | — | — | — |
| 127 | GND |  | GND |  |
| 129 | HDMI_D2_N | — | — | — |
| 131 | HDMI_D2_P | — | — | — |
| 133 | GND |  | GND |  |
| 135 | CAM1_DP3 | — | — | — |
| 137 | CAM1_DN3 | — | — | — |
| 139 | GND |  | GND |  |
| 141 | CAM1_DP2 | — | — | — |
| 143 | CAM1_DN2 | — | — | — |
| 145 | GND |  | GND |  |
| 147 | CAM1_CP | — | — | — |
| 149 | CAM1_CN | — | — | — |
| 151 | GND |  | GND |  |
| 153 | CAM1_DP1 | — | — | — |
| 155 | CAM1_DN1 | — | — | — |
| 157 | GND |  | GND |  |
| 159 | CAM1_DP0 | — | — | — |
| 161 | CAM1_DN0 | — | — | — |
| 163 | GND |  | GND |  |
| 165 | USB_DP | — | USB_DP | USB0+ of LM9514i |
| 167 | USB_DM | — | USB_DM | USB0– of LM9514i |
| 169 | GND |  | GND |  |
| 171 | HDMI_CEC | — | — | — |
| 173 | HDMI_SDA | — | — | — |
| 175 | HDMI_SCL | — | — | — |
| 177 | RUN | — | — | — |
| 179 | VDD_CORE | — | — | DO NOT CONNECT! |
| 181 | GND |  | GND |  |
| 183 | 1V8 | — | 1V8 | +1.8 Vdc |
| 185 | 1V8 | — | 1V8 | +1.8 Vdc |
| 187 | GND |  | GND |  |
| 189 | VDAC | — | 3V3 | +3.3 Vdc, because DAC not in use |
| 191 | 3V3 | — | 3V3 | +3.3 Vdc |
| 193 | 3V3 | — | 3V3 | +3.3 Vdc |
| 195 | GND |  | GND |  |
| 197 | VBAT | — | 3V3 | +3.3 Vdc |
| 199 | VBAT | — | 3V3 | +3.3 Vdc |

## Even numbered PINs on DDR Connector of Raspberry Compute Module

| Pin # | Raspberry Designation | Configuration | Function Designation = Net Name | Peripheral Device |
| --- | --- | --- | --- | --- |
| 2 | EMMC_DISABLE_N | — |  |  |
| 4 | NC | — | — | — |
| 6 | NC | — | — | — |
| 8 | GND |  | GND |  |
| 10 | NC | — | — | — |
| 12 | NC | — | — | — |
| 14 | GND |  | GND |  |
| 16 | NC | — | — | — |
| 18 | NC | — | — | — |
| 20 | GND |  | GND |  |
| 22 | NC | — | — | — |
| 24 | NC | — | — | — |
| 26 | GND |  | GND |  |
| 28 | GPIO28 | — |  | Input 1, needs pull-down resistor |
| 30 | GPIO29 | — |  | Input 2, needs pull-down resistor |
| 32 | GND |  | GND |  |
| 34 | GPIO30 | — |  | Input 3 |
| 36 | GPIO31 | — |  | Input 4 |
| 38 | GND |  | GND |  |
| 40 | GPIO0-27_VDD | — | 3V3 | +3.3 Vdc |
| KEY |  |  |  |  |
| 42 | GPIO28-45_VDD | — | 3V3 | +3.3 Vdc |
| 44 | GND |  | GND |  |
| 46 | GPIO32 | ALT5_TXD1 | RFID2_TXD | UART1, RFID2 |
| 48 | GPIO33 | ALT5_RXD1 | RFID2_RXD | UART1, RFID2 |
| 50 | GND |  | GND |  |
| 52 | GPIO34 | — | OUT1 | Turnstile LED Control |
| 54 | GPIO35 | — | OUT2 | Turnstile LED Control |
| 56 | GND |  | GND |  |
| 58 | GPIO36 | — | OUT3 | Turnstile LED Control |
| 60 | GPIO37 | — | OUT4 | Turnstile LED Control |
| 62 | GND |  | GND |  |
| 64 | GPIO38 | — | OUT5 | Turnstile LED Control |
| 66 | GPIO39 | — | OUT6 | Turnstile LED Control |
| 68 | GND |  | GND |  |
| 70 | GPIO40 | — | OUT7 | Turnstile Passage Control |
| 72 | GPIO41 | — | OUT8 | Turnstile Passage Control |
| 74 | GND |  | GND |  |
| 76 | GPIO42 | — | OUT9 | Turnstile Passage Control |
| 78 | GPIO43 | — | OUTA | Turnstile Auxiliary Control |
| 80 | GND |  | GND |  |
| 82 | GPIO44 | — | PullLow1 | needs pull-down resistor |
| 84 | GPIO45 | — | PullLow2 | needs pull-down resistor |
| 86 | GND |  | GND |  |
| 88 | HDMI_HPD_N_1V8 | — | — | — |
| 90 | EMMC_EN_N_1V8 | — | — | — |
| 92 | GND |  | GND |  |
| 94 | DSI1_DP0 | — | — | — |
| 96 | DSI1_DN0 | — | — | — |
| 98 | GND |  | GND |  |
| 100 | DSI1_CP | — | — | — |
| 102 | DSI1_CN | — | — | — |
| 104 | GND |  | GND |  |
| 106 | DSI1_DP3 | — | — | — |
| 108 | DSI1_DN3 | — | — | — |
| 110 | GND |  | GND |  |
| 112 | DSI1_DP2 | — | — | — |
| 114 | DSI1_DN2 | — | — | — |
| 116 | GND |  | GND |  |
| 118 | DSI1_DP1 | — | — | — |
| 120 | DSI1_DN1 | — | — | — |
| 122 | GND |  | GND |  |
| 124 | NC | — | — | — |
| 126 | NC | — | — | — |
| 128 | NC | — | — | — |
| 130 | NC | — | — | — |
| 132 | NC | — | — | — |
| 134 | GND |  | GND |  |
| 136 | CAM0_DP0 | — | — | — |
| 138 | CAM0_DN0 | — | — | — |
| 140 | GND |  | GND |  |
| 142 | CAM0_CP | — | — | — |
| 144 | CAM0_CN | — | — | — |
| 146 | GND |  | GND |  |
| 148 | CAM0_DP1 | — | — | — |
| 150 | CAM0_DN1 | — | — | — |
| 152 | GND |  | GND |  |
| 154 | NC | — | — | — |
| 156 | NC | — | — | — |
| 158 | NC | — | — | — |
| 160 | NC | — | — | — |
| 162 | NC | — | — | — |
| 164 | GND |  | GND |  |
| 166 | TVDAC | — | — | TV composite output, do not connect |
| 168 | USB_OTGID | — | GND | OTG Pin detect |
| 170 | GND |  | GND |  |
| 172 | VC_TRST_N | — | — | — |
| 174 | VC_TDI | — | — | — |
| 176 | VC_TMS | — | — | — |
| 178 | VC_TDO | — | — | — |
| 180 | VC_TCK | — | — | — |
| 182 | GND |  | GND |  |
| 184 | 1V8 | — | 1V8 | +1.8 Vdc |
| 186 | 1V8 | — | 1V8 | +1.8 Vdc |
| 188 | GND |  | GND |  |
| 190 | VDAC | — | 3V3 | +3.3 Vdc, because DAC not in use |
| 192 | 3V3 | — | 3V3 | +3.3 Vdc |
| 194 | 3V3 | — | 3V3 | +3.3 Vdc |
| 196 | GND |  | GND |  |
| 198 | VBAT | — | 3V3 | +3.3 Vdc |
| 200 | VBAT | — | 3V3 | +3.3 Vdc |

# Ressources

https://elinux.org/RPi_BCM2835_GPIOs

https://pi4j.com/1.2/pins/model-cm3-plus-rev1.html

https://www.raspberrypi.org/forums/

https://de.wikipedia.org/wiki/Raspberry_Pi

https://pi4j.com/1.2/pins/model-cm3-plus-rev1.html

https://en.wikipedia.org/wiki/USB_On-The-Go

# Revision History

| Version | Date | Notes |
| --- | --- | --- |
| 0.1 | 2019-12-03 | Initial draft with new mainboard |
| 1.0 | 2020-02-07 | First Release |
| 1.1 | 2020-02-24 | Illustration of Hardware added |
| 1.2 | 2020-03-12 | Details changed |
| 1.3 | 2020-05-26 | WiFi on Board removed (GPIO21…27 unused), Header for external LCD added, Miscellaneous Interfaces removed, first note of chapter “Housing” added, SD/TF card slot changed to correct pins. |
| 1.4 | 2020-06-03 | Renamed to ID Control 500, Realtek 8188-EU WiFi module added. |

# Technical Data ·  Preliminary

| Electrical Specifications |  |
| --- | --- |
| Power Supply | +5 Vdc |
| Power Consumption | Depending on RFID hardware and devices at USB ports |
| Interfaces | 1 × Ethernet, 3 × USb |
| Trigger Input | Opto-coupler PS2501-1, 1.2 V forward voltage @ 10 mA, external resistor needed! |
| Signal Output | NPN Open-Collector SS8050D, 25 V max, 1.5 A max |

| Mechanical Specifications |  |
| --- | --- |
| Dimensions |  |
| Weight |  |
| Material |  |

| Applicable Standards |  |
| --- | --- |
| EMC | EN 301489-1:2012-04 (v1.9.21) EN 301489-3:2013-12 (V1.6.1) |
| Radio Regulation | EN 300330-1:2015-08 (V1.8.1) EN 300330-2:2015-08 (V1.6.1) |
| Safety | EN 60950-1:2014-08 EN 62369-1:2010-03 EN 50364:2010-11 |
| RoHS, REACH | EU Guideline 1907/2006, updated by 2018/2005/EU (REACH) EU Guideline 2011/65/EU (RoHS 2) EU Guideline 2015/863/EU (RoHS 3) |

| SDK Information |  |
| --- | --- |
| Supported OS | Windows XP, Vista, 7, 8, 8.1, 10 |
| Supported Languages | ASCII command protocol, VS2005 C++ |
| Demo Software | Windows |

Other functions and details to be continued and upgraded.